Table S2. Mesh convergence study showing the effect of mesh resolution on predicted 7-day mass loss. Relative errors are calculated with respect to the reference mesh (21.25 million elements).

| Fidelity | Elements | Mass loss (%) | Relative error (%) |
| --- | --- | --- | --- |
| L1 (Very Low) | 182,768 | 2.5172 | 1465.8 |
| L2 (Low) | 802,365 | 0.8707 | 441.6 |
| L3 (Medium) | 3,454,723 | 0.2857 | 77.7 |
| L4 (High) | 4,990,080 | 0.2112 | 31.4 |
| L5 (Very High) | 11,958,220 | 0.1752 | 9.0 |
| L6 (Ultra High) | 18,864,900 | 0.1622 | 0.87 |
| Reference | 21,245,850 | 0.1608 | — |

Table S3. Comparison between an adaptive mesh (hmin = 0.1 mm, hmax = 2 mm) and a uniform mesh (1 mm everywhere).

| Mesh type | Elements | Mass loss (%) | Error vs. reference (%) | Improvement |
| --- | --- | --- | --- | --- |
| Uniform (1 mm) | ~229,000 | 4.9962 | 3007.8 | Baseline |
| Adaptive (hmin 0.1 mm, hmax 2 mm) | 802,365 | 0.8707 | 441.6 | 0.8532 |
| Reference mesh | 21,245,850 | 0.1608 | — | 1 |

Basis: errors unchanged from the original table; mass loss = reference × (1 + error), anchored so that L6 (18.9M) = 0.1622% (your fine curve at 168 h), giving reference = 0.1608%. NOTE: with these errors L2 (800K) = 0.871%, which contradicts your 800K curve (0.1721% at 168 h); both cannot hold with the errors unchanged.